# Vehicle-to-Vehicle Communication for Blind Spot Detection and Accident Prevention Using Severity-Gated Collision Risk Indexing

Omkar Kadam1, Abhinav Prasad1, Gauri Phanse1, Afraz Khan1, Vaishali Khairnar1, Safia Sadruddin2, Deepali Vora3*

1Department of Information Technology, Terna Engineering College, Nerul, Navi Mumbai, India. 2Department of Computer Engineering, Terna Engineering College, Nerul, Navi Mumbai, India. 3Symbiosis Institute of Technology, Symbiosis International (Deemed University), Pune, 412115, India.

*Corresponding author(s). E-mail(s): deepali.vora11@sitpune.edu.in;

Abstract

Blind spot accidents during lane changes are a major cause of urban traffic collisions. Current driver-assistance systems using radar or cameras fail when other vehicles block the line of sight. This paper presents a Vehicle-to-Vehicle (V2V) Blind Spot Detection system using wireless communication to detect nearby vehicles even when hidden from sensors. The system uses only 5 data fields from the standard Basic Safety Message (BSM) — far fewer than the 13 fields required by previous systems. We introduce a Collision Risk Index (CRI) with a severity gate that prevents false alarms by requiring at least one danger signal to be genuinely high before triggering a warning. Tested over 146,051 simulated vehicle interactions on a real road network, the physics model achieves an

Area Under the Receiver Operating Characteristic curve (AUC) of 0.971 with 95% bootstrap confidence interval [0.956, 0.982] against a model-independent SUMO collision ground truth (n = 27), statistically significantly above all four tested baselines (paired bootstrap, p < 0.001). On a complementary kinematic near-miss proxy the AUC is 0.989 with Area Under the Precision-Recall Curve (AUPRC) = 0.677 against a 0.024 random baseline. Results are stable across six independent random traffic seeds (σ = 0.0041 on AUC). An exploratory XGBoost classifier and three additional learners (LightGBM, Random Forest, Logistic Regression) are included as data-driven baselines.

Keywords: Blind Spot Detection, Collision Risk Index, DSRC/C-V2X, Severity Gate, Vehicle-to-Vehicle Communication

# Introduction

Road traffic injuries cause about 1.19 million deaths worldwide every year, making it the leading cause of death for people aged 5–29 [10]. Many of these acci-dents happen when drivers change lanes without seeing vehicles in their blind spots. In India, dense and mixed traffic on urban roads makes the problem worse [11].

The canonical failure mode is a lane-change manoeuvre in heavy traffic, illustrated in Fig. 1. The

ego vehicle (a sedan in the middle lane) signals a merge into the adjacent right-hand lane. A heavy truck travelling alongside in the ego’s own lane physically occludes the ego’s onboard radar and camera, hiding a faster sedan that is approaching from behind in the des-tination lane. The hidden vehicle sits squarely inside the ego’s right-side blind spot. The driver, receiving no sensor alert, initiates the lane change, and the rear-end speed difference leaves the approaching vehicle no time

to brake. Side-swipe collisions of this kind account for a non-trivial share of urban lane-change crashes [14], and they cannot be eliminated by improving sensor resolution alone — the line-of-sight is geometrically blocked.

Vehicle-to-Vehicle (V2V) communication addresses this gap by letting vehicles exchange short safety messages over radio links that propagate around obstructions. V2V is not intended as a substitute for radar or camera sensing; it is a third sensing modality, used in combination with the on-board sensors to cover precisely those situations where line-of-sight is geometrically impossible. Each vehicle broadcasts its location, speed, and acceleration as a Basic Safety Message (BSM) [1, 2] at 10 Hz, with an effective range of up to roughly 300 m on either Dedicated Short-Range Communication (DSRC, IEEE 802.11p) [3] or Cellular V2X (C-V2X, 3GPP) [4].

Previous V2V blind-spot systems consume up to 13 BSM fields, including heading angle, turn signal indi-cators, and yaw rate [12]. Requiring those extra fields excludes vehicles whose on-board units broadcast only the mandatory Part I message, and it creates a hard dependency on CAN-bus access for signals such as the turn indicator. The system presented here uses only five fields — GPS position (x, y), speed (v), accelera-tion (a), and braking magnitude (d) — and derives the remaining quantities (heading, yaw rate, lane-change intent) from position history.

The main contributions are:

- Severity-gated Collision Risk Index. A risk for-mula requiring at least one genuinely high danger signal before triggering a warning, preventing false alarms from small values adding up.

- Minimal 5-field BSM. Eliminates the need for 13 fields, making the system compatible with all vehicle types.

- Exploratory ML baseline comparison. An XGBoost classifier reveals that physics models out-perform data-driven approaches when dangerous events are rare.

- Realistic testing. Tested on a real road network with traffic signal violations, narrow hilly roads, and simulated communication failures.

# Related Work

Research on blind-spot detection has converged from three originally separate communities — sensor-based

driver-assistance systems, V2V communication pro-tocols, and learning-based risk assessment — and a fourth, C-V2X cooperative perception, has emerged more recently. This section covers each in turn.

## Sensor-Based and V2V Blind Spot Detection

Current commercial systems use radar sensors on rear bumpers, which work well on open roads but fail in dense traffic where large vehicles block signals. V2V communication overcomes this as radio signals pass through vehicles. V2V systems use SAE J2735 [1] for message formatting and IEEE 802.11p [3] for trans-mission. The SAE J2945/1 standard [2] requires 10 Hz BSM broadcasts. Our system uses only 5 BSM Part I fields, compatible with basic on-board units [13].

## Risk Assessment and Machine Learning

Time-to-Collision (TTC) is widely used in Euro NCAP testing but becomes unreliable with lateral vehicle movement. Early V2V systems used shared GPS coor-dinates; Kim et al. developed threat assessment using V2V data [16]; Mousa et al. created a 13-field V2V blind spot system [12]. A common problem is that mul-tiple small risk values trigger false warnings — our severity gate prevents this. Machine learning models like XGBoost work well for structured data, but raise interpretability concerns for safety decisions. The Syn-thetic Minority Over-sampling Technique (SMOTE) helps handle rare dangerous events by interpolat-ing new minority-class samples in feature space. In our system, an XGBoost classifier is included as an exploratory comparison only.

## C-V2X Cooperative Perception

A more recent line of work moves beyond single-vehicle BSM exchange to full cooperative perception, in which connected vehicles share raw or feature-level sensor data over the C-V2X link to extend each other’s perceptual range. OPV2V [19] introduced an open simulation benchmark with multi-vehicle LiDAR fusion. V2X-ViT [20] extended this to a vision-transformer-based fusion backbone that is robust to localisation noise. V2V4Real [21] contributed the first real-world multi-vehicle perception dataset, col-lected on public roads. V2X-Sim [22] formalises the multi-agent collaborative-perception setting as a bench-mark with both BEV-detection and segmentation tasks.

Fig. 1 Lane-change blind-spot scenario. An occluding truck blocks the ego’s onboard sensors, but the 5-field BSM from the target reaches the ego over the V2V radio link. The receiver feeds the BSM into the Collision Risk Index (CRI) engine, which applies a severity gate and raises a WARNING alert before the lane change completes.

Two recent surveys give a comprehensive overview of the design space [23, 24], and Garlichs et al. [25] discuss the standardisation considerations for produc-tion C-V2X. The work presented here is intentionally complementary to that body of literature: it operates entirely within the minimum BSM Part I envelope, with no shared sensor data, no bandwidth-heavy LiDAR sweeps, and no neural-network-scale inference bud-get. That makes it deployable on the simplest legal on-board unit, but it also means it does not solve the broader cooperative-perception problem — cooperative perception adds non-line-of-sight detection of non-V2V-equipped traffic, which 5-field BSM exchange cannot do.

# System Architecture

The system is organized as a four-layer pipeline Fig. 2: Layer 1 (Simulation): Eclipse SUMO generates vehi-cle movements at 10 Hz with injected danger scenarios. Layer 2 (Communication): Vehicle data is packaged into 5-field BSMs with GPS noise (1.5 m) and packet drops (Gilbert-Elliott model). Lost messages trigger dead-reckoning prediction. Layer 3 (Intelligence): Two parallel paths process received data — a physics model (primary, CRI-based) and an XGBoost classifier (exploratory baseline). Layer 4 (Analytics): A Stream-lit dashboard displays real-time V2V connections and risk scores.

Fig. 2 Four-layer V2V BSD system architecture.

# Mathematical Model

The system takes 5 data fields from each nearby vehi-cle and produces a risk score between 0 (safe) and 1 (critical).

## BSM Input and Heading Estimation

The 5-field BSM is BSM = {x, y, v, a, d}, where x and y are GPS coordinates (converted to metres in a local UTM frame), v is the scalar speed, a is the positive

part of the signed acceleration, and d is the positive part of the deceleration. Heading is not transmitted; it is recovered from two consecutive positions using the

four-quadrant inverse tangent:

θ (t) = atan2 y(t) − y(t − 1), x(t) − x(t − 1)	(1)

This means the heading vector is the direction from the previous reported position to the current one, which equals the true heading when the vehicle is moving and the GPS noise is small relative to the displace-ment between samples. At speeds below 0.5 m/s the

## Blind Spot Zone Probability

Since GPS has 1.5 m uncertainty, the system calculates the probability of a target being in the blind spot using Gaussian integration:

σ	σ

Plat = Φ  xmax − xc  − Φ  xmin − xc	(5)

Plon = Φ  ymax − yc  − Φ  ymin − yc	(6)

displacement between consecutive samples can be	σ	σ

smaller than the GPS noise itself, so θ (t − 1) is held

instead of being recomputed — otherwise GPS jitter would rotate the heading by tens of degrees while the vehicle is essentially stationary. The yaw rate is then

θ˙(t) = [θ (t) − θ (t − 1)]/∆t with ∆t = 0.1 s.

## Coordinate Transformation and Blind Spot Zone

The target’s position is transformed into the ego vehicle’s reference frame:

xrel = sin(θe) · ∆x − cos(θe) · ∆y	(2)

yrel = cos(θe) · ∆x + sin(θe) · ∆y	(3)

where ∆x = xt − xe, ∆y = yt − ye. The blind spot zone extends with speed:

L	v	L	λ	clamp  ve − vmin  0 1

where Φ is the standard normal cumulative distribution function (CDF) and σ = 1.5 m. The combined proba-bility is Pgps = |Plat| × Plon, with a forward-lane filter to prevent false alarms from vehicles directly ahead.

## Dead Reckoning and Packet Loss

When packets are lost, the system predicts target position using a Constant-Acceleration, Constant-Yaw-Rate model:

2

xˆ = x + v cos(θ ) τ + 1 anet cos(θ ) τ2	(7)

2

yˆ = y + v sin(θ ) τ + 1 anet sin(θ ) τ2	(8) Prediction is limited to 0.5 s; targets are dropped after four consecutive lost packets. A Packet Loss Ratio

(PLR) tracked over a 10-step window increases risk: Γplr = 1 + 0.30 · PLRwindow.

bs( e) =

base +

scale ·

vmax − vmin , ,

(4)

  - Deceleration Risk (Rdecel)

rel

where Lbase = 4.5 m is the minimum zone length and λscale = 12.0 m is the speed-dependent extension. On curves, a curvature correction shifts the zone laterally: xcorrected = xrel − y2 θ˙e/(2ve).

This component measures whether the target vehicle can stop in time. The maximum braking capability combines tyre-road friction and aerodynamic drag:

Ego vehicle class. The zone parameters Lbase = 4.5 m and λscale = 12.0 m correspond to a passenger-car ego (sedan/SUV), whose body length is approximately

amax =

µ · g +

ρ · Cd · Af · v2

t

2 · Mt

(9)

one base-zone unit and whose A/B/C-pillar geome-try produces a rear-quarter blind spot of roughly the dimensions given. The simulated fleet does include trucks as target vehicles, but the CRI engine reported in this paper treats only passenger cars as the ego — the entity whose blind spot is being monitored. For a truck ego, Lbase would scale to the vehicle length (around 12–14 m) and λscale would expand to reflect the much larger rear-and-side blind region around an articulated heavy vehicle. The same equations apply; only the con-stants change. Per-class parameter calibration for heavy vehicles is left for future work.

where µ is the tyre-road friction coefficient, g is grav-itational acceleration, ρ is air density, Cd is the drag coefficient, Af is the frontal area, vt is the target speed, and Mt is the target mass. The friction coefficient µ is treated as a per-scenario constant rather than a continuously varying weather-adaptive parameter:

µ = 0.70 for the baseline dry-asphalt scenario and µ =

0.55 for the Hilly Narrow Road scenario (Section 5). A dynamic, weather-coupled friction model would

require sensor inputs (wheel-slip estimates, ambient temperature) that the 5-field BSM does not carry; we treat real-time friction adaptation as future work. The

total stopping distance includes a 1.2 second reac-tion time consistent with AASHTO 85th-percentile guidance [6]:

v2

the upper bound over which a lane-change manoeuvre would still plausibly bring the two trajectories together; beyond 8 s the predicted impact is too far in the future to motivate an alert, since intervening driver actions and updated BSMs are nearly certain to invalidate the

Dstop = vt · Treact + 2

t

· amax

(10)

prediction. The quadratic decay between 4 s and 8 s is chosen so that the score falls smoothly from 1.0

The risk grows exponentially as the stopping distance approaches the actual gap between vehicles:

to 0.25 as the time horizon doubles, which is consis-tent with the conservative roll-off used in Euro NCAP surrogate-safety testing.

Rdecel = clip

[0,1]

exp −Kbrake · dgap − Dstop  A parallel calculation evaluates side-impact (sideswipe)

where clip[0,1](z) = min 1, max(0, z)

Dstop

(11)

geometry using the lateral gap and lateral closing rate, producing Rttc,lat. For the lateral component a lin-

ear decay 1 − TTClat/TTCcrit is used instead of the

with Kbrake = 1.50. The notation clip[0,1](·) simply means the expression is bounded between 0 and 1 so

Rdecel remains a valid probability-style score even at the extremes. When the gap exactly equals the stop-ping distance, Rdecel = 1.0; as the gap grows, the value decays toward zero.

## Time-to-Collision Risk (Rttc)

quadratic form, because lateral closing speeds are typ-ically much smaller than longitudinal ones and the quadratic form would suppress almost all lateral signal. The combined component reported as Rttc in the CRI formula is the higher of the two values:

Rttc ← max Rttc,lon, Rttc,lat .	(14)

The Time-to-Collision risk component captures how imminent a closing manoeuvre is, expressed on a 0–1 scale. The Time-to-Collision TTC itself is the smaller positive real root τ of the longitudinal kinematic equation

arel · τ2 + vrel · τ + xrel = 0,	(12)

  - Intent Risk (Rintent)

The 5-field BSM does not carry the turn-signal indica-tor, so lane-change intent cannot be read directly from the source vehicle. Two indirect signals are available: the lateral component of the ego velocity (computed from successive position samples), and the heading change rate θ˙. The system uses the lateral velocity:

where xrel is the current longitudinal gap between ego and target (metres), vrel is the relative closing speed in the ego direction (m/s, positive when the gap is shrinking), and arel is the relative acceleration (m/s2). If

Rintent

where vlat

= 0.6 · min 1, max(0, vlat,toward) ,	(15)

1.0 m/s

is the component of the ego’s lateral

no positive real root exists, the two vehicles will never meet on the current trajectory and TTC is defined as

+∞. The risk is then a piecewise function of TTC:

1.0	if TTC ≤ TTCcrit,

,toward

velocity directed toward the target’s lane. Choosing

the lateral-drift indicator over an unobservable turn-signal field has a useful safety property: if the human driver forgets to activate the indicator before merging

— a frequent real-world failure mode — the lateral

TTC

Rttc

=  TTCcrit 2



if TTCcrit < TTC ≤ TTCmax,

drift component still detects the manoeuvre, because the actual lane change physically produces a non-zero vlat toward at least a fraction of a second before the

0	otherwise.

(13)

,

wheels cross the lane line. Empirically, however, this

lateral-only proxy generates too many false positives

Threshold selection. The two breakpoints are not arbi-trary. The critical threshold TTCcrit = 4.0 s matches the NHTSA Forward Collision Warning band, in which a TTC at or below 4 s combined with a non-zero clos-ing rate is treated as an imminent-warning condition for passenger vehicles. The horizon TTCmax = 8.0 s is

during normal in-lane jitter and curved-road tracking. Test runs over the validation split repeatedly trig-gered intent risk during routine driving on the bridge approach curves. The component is therefore disabled

in the published configuration by setting γ = 0, and the lane-change danger case is captured by the deceleration

and TTC components only (which respond directly to the closing geometry that an intended merge creates). The distributions of all three risk components across the simulation are shown in Fig. 3. High-risk val-ues are rare, consistent with the 2.37% positive event

rate.

Fig. 3 Distributions of the three CRI risk components. High-risk values are rare, consistent with the 2.37% positive event rate in the simulation.

## CRI with Severity Gate

The final Collision Risk Index combines all compo-nents:

Fig. 4 CRI score distribution across 146,051 timesteps. 96.0% remain in the SAFE region, confirming the severity gate prevents excessive warnings.

single vehicle pair, showing how the hysteresis mech-anism prevents alert flickering during risk escalation and de-escalation.

CRI = clip Pgps · max(Rdecel, Rttc)

- αRdecel + β Rttc + γRintent

- Γplr, 0, 1

(16)

Fig. 5 Mean alert level over the 360-second simulation. Spikes correspond to injected danger scenarios (traffic signal violations and

The max(Rdecel, Rttc) severity gate prevents false alarms from small harmless values adding up by requiring at least one genuinely high danger signal. Optimized weights: α = 0.20, β = 0.80, γ = 0.00.

## Alert Levels

The system triggers four alert levels based on CRI:

    - SAFE (CRI < 0.30): No danger detected

    - CAUTION (CRI ≥ 0.30): Vehicle detected in blind spot

    - WARNING (CRI ≥ 0.60): Close proximity or approach

    - CRITICAL (CRI ≥ 0.80): Imminent collision risk

To prevent alerts from flickering on and off, the sys-tem requires 3 consecutive steps above a threshold before upgrading, and a small buffer band (0.05) before downgrading. Left and right sides are monitored inde-pendently. The CRI distribution in Fig. 4 confirms 96.0% of observations remain SAFE.

Fig. 5 shows the alert activity over the full simu-lation, with spikes corresponding to injected danger scenarios. Fig. 6 illustrates the CRI time-series for a

narrow road events).

Fig. 6 CRI time-series for a single vehicle showing risk escalation and de-escalation. Horizontal lines indicate alert thresholds: CAU-TION (0.30), WARNING (0.60), CRITICAL (0.80). The hysteresis

requirement (3 consecutive steps) delays alert upgrades, preventing flicker.

# Scenario Modelling

To test the system under realistic danger conditions, two conflict scenarios and a communication failure model were implemented.

## Traffic Signal Violation (TSV)

When a vehicle approaches an intersection during a red light, there is a 30% chance it will ignore the signal and continue at full speed. This creates dangerous crossing conflicts that test the system’s ability to detect side-impact collision risks.

## Hilly Narrow Road (HNR)

On selected road segments, lane width is reduced to 2.8 metres and road friction is lowered to 0.55 (simulating wet or hilly conditions). Some vehicles (10% probability) attempt aggressive overtaking. This tests the system’s ability to handle reduced braking capability and tight lateral spaces.

## Communication Packet Loss

Real-world wireless signals can be interrupted by build-ings, other vehicles, or interference. This is modelled using a Gilbert-Elliott two-state channel: in the good state, only 1% of packets are lost; in the bad state, 50% are lost. The system spends about 9.1% of the time in the bad state, creating realistic multi-packet outages that test the dead-reckoning prediction.

The effect of these scenarios on risk levels is shown in Fig. 7. TSV events cause periodic spikes at intersec-tions; HNR conditions cause sustained elevated risk from reduced friction and narrow lanes.

Fig. 7 Mean CRI over time by scenario type. TSV events cause periodic spikes at intersections; HNR conditions cause sustained elevated risk.

# Experimental Setup

The experimental setup is described in five parts: the source and structure of the synthetic dataset, the SUMO simulation environment, the road network with its explicit lane-change traffic generator, the reference table of all system parameters, and the configuration of the exploratory XGBoost classifier.

## Dataset Description

The dataset used in this work is generated synthetically by the Eclipse SUMO microscopic traffic simulator running on a real-world road network — the Atal Setu corridor in Navi Mumbai, India, extracted from Open-

StreetMap (bounding box lat 18.992–19.003°N, lon 72.846–72.862°E, downloaded on 26 September 2025 under ODbL v1.0). The OpenStreetMap road geometry

is converted into a SUMO network with 539 named road segments, 2,103 lanes, and 501 junctions using netconvert v1.24.0 with UTM zone 43 projection. The simulator is run for 3,600 steps of 0.1 s each (360 s of simulated time) at a BSM update rate of 10 Hz, under a fixed random seed of 42. The simulated vehicle fleet contains three classes (sedans 65%, SUVs 25%, trucks 10%) with realistic acceleration and braking pro-files. Three hazard scenarios are tagged in the output: Normal traffic (the SUMO default car-following and lane-changing behaviour), TSV — Traffic Signal Vio-lation (some red-light-running vehicles), and HNR — Hilly Narrow Road (reduced friction and tight lateral spaces). A Gilbert–Elliott two-state Markov channel injects realistic packet loss on the V2V link. The result-ing dataset contains 146,051 observations, each row consisting of 45 columns: kinematic state, CRI sub-components, packet-loss indicators, alert levels for the physics model and two baselines, and the XGBoost prediction. The class composition by scenario tag is

Normal = 49.6%, HNR = 31.0%, and TSV = 19.3%.

These are scenario row counts, not positive-event rates

— the positive-event rate (kinematic near-miss or physi-cal collision) is much smaller, 2.37%, across the whole dataset.

## Simulation Environment

All experiments use Eclipse SUMO running at 10 Hz. The road network is a 539-edge map of the Atal Bridge corridor in Navi Mumbai, India, extracted from Open-StreetMap. The simulation runs for 360 seconds of simulated time and peaks at 163 simultaneous vehi-cles, yielding 146,051 total observations under random seed 42 (deterministic; same seed produces identical telemetry on the same SUMO version).

## Road Network and Lane-Change Setup

The Atal Bridge network includes multi-lane roads, controlled intersections, and highway ramps. The

traffic generation script forces dense lane-change inter-actions throughout the network using a four-phase route generator. Phase 1, which is the primary source of blind-spot events, populates every multi-lane edge with paired vehicles every 3.0 s of simulated time: a slow leader vehicle is inserted on lane 0 at 30 m from the edge start, travelling at 12 m/s, and an aggres-sive catcher vehicle is inserted on lane 1 at the edge start, travelling at 28 m/s with maxSpeed 35 m/s. The lane assignment alternates per timestep so both left-side and right-side blind-spot configurations are generated. Where three lanes are available, a third truck-type vehicle on lane 2 acts as an occluder for line-of-sight sensing. Phase 2 adds 150 cross-network commuter routes; Phase 3 inserts platoons of six vehi-cles at 0.5 s headway; Phase 4 streams vehicles inward from every network-boundary edge. SUMO’s built-in LC2013 lateral controller produces the actual lane-change dynamics, with lateral resolution set to 0.8 m

and speed deviation 0.1 on each vehicle type. Lane-change events triggered by this combination of forced insertion patterns and the LC2013 model are what gen-

erate the 3,465 positive ground-truth events used to score the CRI.

## Parameter Reference Table

The system parameters are listed in Table 1.

## AI Model Configuration

An XGBoost classifier [8] serves as an exploratory baseline. Fifteen features are computed from the 5 BSM fields, of which approximately 11 are lin-early independent once correlated pairs (e.g., speed and speed_kmh) are removed. The dataset is heavily skewed: out of 146,051 observations only 54 carry a CRITICAL label, which is a positive rate below 0.04%. Training on the raw distribution would let the classi-fier collapse to “always predict SAFE” and still reach 99.96% accuracy. To prevent that, SMOTE (Synthetic Minority Over-sampling Technique) [15] is applied to the training partition only. SMOTE picks a minority-class sample, finds its k nearest minority neighbours in feature space, and synthesises a new sample on the line segment connecting them. The CAUTION class is oversampled to 3,000 rows, WARNING to 2,000 rows, and CRITICAL to 500 rows — enough to push gradient updates toward the rare-event decision bound-ary without copying any duplicate rows. The held-out 25% test partition is never SMOTE-treated, so reported test metrics reflect the true class distribution. We use

Table 1 System Parameter Reference

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| BSM Frequency | 1/∆t | 10 | Hz |
| Detection Range | Rmax | 300 | m |
| GPS Uncertainty | σgps | 1.5 | m |
| Reaction Time | Treact | 1.2 | s |
| Baseline Friction | µ | 0.70 | – |
| Lane Width | Wlane | 3.5 | m |
| Critical TTC | TTCcrit | 4.0 | s |
| Maximum TTC | TTCmax | 8.0 | s |
| Safe Boundary | θ1 | 0.30 | – |
| Warning Boundary | θ2 | 0.60 | – |
| Critical Boundary | θ3 | 0.80 | – |
| Decel. Weight | α | 0.20 | – |
| TTC Weight | β | 0.80 | – |
| Intent Weight | γ | 0.00 | – |
| Brake Decay | Kbrake | 1.50 | – |
| PLR Penalty | ε | 0.30 | – |
| Hysteresis Steps | NH | 3 | – |
| Downgrade Band | δh | 0.05 | – |
| Lateral Tolerance | ∆W | 0.5 | m |
| Low-Speed Guard | vmin,θ | 0.5 | m/s |
| Good→Bad prob. | pG→B | 0.01 | – |
| Bad→Good prob. | pB→G | 0.10 | – |

5-fold time-series cross-validation for internal stability checks, on top of the 75/25 stratified holdout split.

# Results and Analysis

This section reports the evaluation in the order a reader needs to consume it: first the ground-truth defi-nition used by every following metric, then the overall performance table with confidence intervals and signif-icance tests, then a held-out calibration check that rules out test-set leakage, and finally the ROC/PR curves, the exploratory ML comparison, the per-scenario and multi-seed robustness checks, the ablation, and the sensitivity analysis.

## Ground Truth Definition

Two complementary ground-truth sources are used. We refer to them as Tier 1 and Tier 2 throughout the paper; the labels refer to the source of the positive label, not to road friction or any other physical quantity.

#### Tier 1 — SUMO physical collisions (model-independent).

At each simulation step, SUMO’s collision detector reports every pair of vehicles that physically overlap. Both the collider and the victim are flagged as positive for that step. Across the 360 s run there are 27 such collision events. These labels are produced entirely by SUMO’s microscopic dynamics and do not use any quantity computed by the CRI engine, so they constitute an independent ground truth.

#### Tier 2 — Kinematic near-miss proxy.

Physical collisions are extremely rare in any well-behaved simulation, which leaves very few samples for receiver-operating-characteristic estimation. To supplement Tier 1, a deterministic kinematic rule flags additional near-miss events using only quantities present in the BSM. A row is labelled positive when all of the following hold:

- the ego speed exceeds 2.0 m/s (filters parked or stationary egos);

- the target lies within the blind-spot zone (in_zone_left or in_zone_right is true);

- either the longitudinal gap is below 1.0 m, or the kinematic TTC proxy TTCproxy = dgap/ max(vrel, 0.5 m/s) is below 2.0 s;

- the relative speed exceeds 0.5 m/s (avoids division-

by-near-zero in the TTC proxy).

The thresholds are not free hyperparameters. The 1.0 m gap is a vehicle-width safety margin (a gap below one vehicle width during a lane-change implies side-swipe contact in the next sample). The 2.0 s TTC thresh-old corresponds to the imminent-warning band used in the AEB regulatory test protocols. The 0.5 m/s relative-speed guard prevents the proxy from triggering on stationary co-travellers where the TTC denomi-nator would collapse. The 2.0 m/s ego-speed guard

mirrors the parameter vmin = 2.0 m/s used by the dynamic blind-spot zone (Section 4). Combining Tier 1

and Tier 2 gives 3,465 positive events on 146,051 observations, a positive rate of 2.37%.

#### Caveat (acknowledged honestly).

The Tier 2 proxy uses some of the same input quantities (gap, TTC) that the CRI engine consumes. As a result, the Tier 2 AUC measures rank-ordering consistency between the CRI score and the same set of underlying physical signals — it is not an independent validation. The model-independent AUC against the 27 Tier 1

collisions is reported separately throughout this paper; it has wider confidence intervals because of the small sample, but it does not share inputs with the CRI score.

## Overall Performance

The system was evaluated over 146,051 observations split by scenario as Normal 49.6%, HNR 31.0%, TSV 19.3% (these percentages are scenario row counts; the positive-event rate is 2.37% across all rows). Perfor-

mance is reported in Table 2 against both ground-truth sources defined above. The order of columns puts the model-independent SUMO-collision metric first, because it is the one that does not share inputs with the CRI score. All AUC values carry 95% confidence intervals computed by stratified bootstrap over 500 resamples of the test partition. Pairwise differences in AUC between the physics model and each baseline are tested by paired bootstrap on the same resamples (sig-

nificance codes: ∗ p < 0.05, ∗∗ p < 0.01, ∗∗∗ p < 0.001, two-sided).

The physics model is statistically better than every baseline on both ground-truth definitions. Critically, the Tier 1 (model-independent) ranking is identical to the Tier 2 (proxy) ranking: the proxy is not inverting any conclusions. The relative gap is larger on Tier 1

(Physics − TTC = +0.17 AUC) than on Tier 2 (Physics

— TTC = +0.10 AUC), which is the opposite of what one would expect if the published numbers were purely

a self-consistency artefact. AUPRC for the physics model is 0.69 on the proxy — 30× the random-baseline AUPRC of 0.024 driven by the 2.37% positive rate.

## Calibration Discipline Verification

A reasonable concern with the headline number is whether the CRI weights α = 0.20, β = 0.80 were selected with knowledge of the test data — in other words, whether they are fitted on the test set rather than learned from training. To rule this out we re-ran the weight selection under a strict 75/25 stratified train/test split (random seed 42), so that no part of the held-out test set is visible during weight selection. The grid search swept α ∈ [0.10, 0.90] in 0.05 steps

with α + β = 1 and γ fixed at zero. Both AUC and

F1 selection criteria identified the same train-optimal weights, α⋆ = 0.90, β ⋆ = 0.10 (a different point from the published configuration). We then evaluated both

weight settings on the same untouched test partition. The results are in Table 3.

The published weights generalise to the held-out test partition at Tier 1 AUC = 0.990 (CI overlaps the

Table 2 Performance with 95% bootstrap CIs and pairwise significance vs. Physics. Tier 1 (SUMO collisions, n = 27) is model-independent; Tier 2 (kinematic proxy) shares inputs with the CRI score and is reported only for completeness.

| Method | Tier 1 AUC [95% CI] | vs. Phys. | Tier 2 AUC [95% CI] | vs. Phys. |
| --- | --- | --- | --- | --- |
| Physics (CRI) | 0.971 [0.956, 0.982] | — | 0.989 [0.987, 0.990] | — |
| TTC kinematic | 0.801 [0.700, 0.892] ∗∗∗ 0.893 [0.885, 0.901] ∗∗∗ |  |  |  |
| Static box | 0.800 [0.698, 0.891] ∗∗∗ 0.875 [0.866, 0.882] ∗∗∗ |  |  |  |
| XGBoost (exploratory) | 0.819 [0.779, 0.861] ∗∗∗ 0.798 [0.792, 0.806] ∗∗∗ |  |  |  |

Table 3 Held-out test performance under the published weights vs. train-only-optimal weights. Both configurations generalise comfortably; the published weights are slightly better on Tier 1, the train-optimal weights are slightly better on Tier 2.

| Weights (α, β ) | Tier 1 AUC [95% CI] | Tier 2 AUC [95% CI] | F1 @ 0.60 | Test rows |
| --- | --- | --- | --- | --- |
| Published (0.20, 0.80) | 0.990 [0.986, 0.996] | 0.989 [0.986, 0.991] | 0.569 | 28,037 |
| Train-optimal (0.90, 0.10) | 0.989 [0.984, 0.996] | 0.994 [0.991, 0.996] | 0.614 | 28,037 |

train-optimal weights almost entirely), so the headline result is not a product of test-set leakage. We keep the published weights α = 0.20, β = 0.80 as the produc-tion configuration because (i) they remain within the bootstrap CI of the train-optimal point, (ii) they per-

form marginally better on the truly independent Tier 1 ground truth, and (iii) re-running the simulation under the alternative weights does not visibly change the alert behaviour in the per-vehicle time-series plots.

## ROC and PR Analysis

Fig. 8 shows the ROC curves for all four methods, computed on the Tier 2 kinematic proxy across the full 146,051-row evaluation set. The Tier 2 ranking matches Table 2: physics model at AUC = 0.989, TTC kinematic baseline at 0.893, static box baseline

at 0.875, and the exploratory XGBoost classifier at 0.798. Against the 27 Tier 1 SUMO collisions, the physics model achieves AUC = 0.971 with 95% CI

[0.956, 0.982]; the gap to every baseline is wider on

Tier 1 than on Tier 2 (Physics − TTC = +0.17 on Tier 1 vs. +0.10 on Tier 2), which is the opposite of what would be expected if the proxy were inflat-

ing the physics-model score by input overlap. The physics model also achieves an AUPRC of 0.677 on the proxy, far above the random baseline of 0.024 driven by the 2.37% positive rate. Fig. 9 shows the precision-recall curve, which is the more informative view when positive events are this rare and ROC-AUC can be misleadingly optimistic.

Fig. 8 ROC curves for four detection methods.

## Exploratory AI Model Results

The XGBoost classifier is included as an exploratory learning-based baseline rather than a proposed deploy-ment component. Its overall ranking depends on the ground-truth source: on the Tier 2 kinematic proxy

it scores AUC = 0.798 (below both TTC and Static box, see Table 2); on the Tier 1 model-independent ground truth it scores AUC = 0.819 (slightly above both kinematic baselines). The proxy-vs-independent

gap is itself informative — it shows that the XGBoost classifier is somewhat over-fit to the proxy’s same-input structure, while the physics model is not. On the per-class breakdown (Table 4) the classifier detects

Fig. 9 Precision-Recall curve (AUPRC=0.677 vs. 0.024 random baseline).

CAUTION (65% recall) and WARNING (57% recall) events but fails entirely on the CRITICAL class (0% recall, 13 test samples). The failure is systematic rather than statistical: five-fold time-series cross-validation also yields 0% CRITICAL recall across all folds, and Section 7.6 shows that Random Forest, LightGBM, and Logistic Regression all hit the same wall under iden-tical pre-processing. The confusion matrix in Fig. 10 visualises the failure pattern.

Feature importance analysis as shown in Fig. 11 shows that the most important features are braking dynamics (decel=0.34) and gap distance (max_gap=0.14). The 15 input features include four perfectly correlated pairs (speed/speed_kmh, rel_speed/closing_speed, accel/abs_accel, decel/ is_braking), as shown in the correlation heatmap in Fig. 12, reducing the effective feature count to approximately 11 unique signals.

Table 4 AI Model Class Performance

| Class | Prec. | Recall | F1 | Support |
| --- | --- | --- | --- | --- |
| SAFE | 0.99 | 0.82 | 0.90 | 35,047 |
| CAUTION | 0.12 | 0.65 | 0.20 | 907 |
| WARNING | 0.11 | 0.57 | 0.18 | 546 |
| CRITICAL | 0.00 | 0.00 | 0.00 | 13 |

Fig. 10 Four-class confusion matrix: XGBoost predictions vs. physics-model CRI labels. CRITICAL detection fails due to extreme class rarity (54 total samples).

Fig. 11 XGBoost feature importance (15 features). Deceleration and gap distance dominate. Scenario labels are excluded from the feature set.

### SHAP-based feature attribution

The gain-based importance reported in Fig. 11 mea-sures how often each feature is used to split a tree node weighted by the loss reduction it produces, which is a model-internal quantity. To produce a per-sample, model-agnostic measure of feature con-tribution we additionally computed Shapley Additive exPlanations (SHAP) values [18] on a stratified 3,791-row sample (approximately 1,250 per class) using the TreeExplainer for XGBoost. Fig. 13 shows the SHAP beeswarm summary for the CRITICAL class, and Fig. 14 shows the dependence plot for the most-impactful feature.

Fig. 12 Feature correlation heatmap. Notable redundancies include speed/speed_kmh (r = 1.0) and rel_speed/closing_speed (r = 1.0), suggesting potential for feature set reduction.

Fig. 13 SHAP beeswarm summary for the CRITICAL class pre-dictions. Each dot is one sample; horizontal position is the SHAP value (impact on the CRITICAL log-odds); colour encodes the fea-ture value (red = high, blue = low). max_gap dominates by a wide margin, and the directionality is the physically expected one: small gaps push the prediction toward CRITICAL.

The gain-based ranking (Fig. 11) and the SHAP ranking agree on the most-impactful features but in a slightly different order: SHAP places max_gap first, then num_targets, speed, decel; gain places decel first followed by is_braking and max_gap. The two views are complementary — gain measures the

Fig. 14 SHAP dependence plot for decel on the CRITICAL class. Increasing deceleration consistently increases the predicted CRIT-ICAL probability; the vertical spread at each decel value reflects interaction with the second most-related feature, which SHAP selects automatically.

model’s reliance on a feature across all training sam-ples; SHAP measures the per-sample contribution to specific predictions. The fact that the two top features by SHAP (max_gap, num_targets) are also the inputs the physics CRI engine treats as primary — gap dis-tance and target presence — is reassuring from an interpretability standpoint.

## Comparison with Other ML Methods

A natural question after observing the XGBoost result is whether the underperformance is specific to gradi-ent boosting or whether any data-driven classifier hits the same wall. To answer this we trained three addi-tional classifiers on the same 15-feature table, the same train/test split, and the same SMOTE pre-processing as the XGBoost model: a Random Forest with 200 trees and balanced class weights, a LightGBM gradient-boosting classifier matched to the XGBoost hyperpa-rameters, and a multinomial Logistic Regression with standardised inputs. Table 5 reports macro-OvR AUC, CRITICAL-binary AUC, macro F1, and CRITICAL recall on the held-out test set.

The three additional classifiers all sit in the same performance band as XGBoost, and all four fail identi-cally on the CRITICAL class. None of them recovers a CRITICAL sample in the held-out test set — not because the algorithms are incapable but because there are only 41 CRITICAL training rows after the strat-ified split (extended to 500 by SMOTE). With this little signal, no classifier can confidently distinguish

Table 5 Held-out test performance of four data-driven classifiers under identical preprocessing. All four models exhibit zero CRITICAL recall, confirming that the rare-event ceiling is a property of the data distribution rather than a particular classifier choice.

| Classifier | Macro AUC | CRITICAL AUC | Macro F1 | CRITICAL Recall |
| --- | --- | --- | --- | --- |
| XGBoost | 0.931 | 0.909 | 0.325 | 0.00 |
| LightGBM | 0.911 | 0.883 | 0.314 | 0.00 |
| Random Forest | 0.908 | 0.850 | 0.352 | 0.00 |
| Logistic Regression | 0.858 | 0.762 | 0.299 | 0.00 |

the CRITICAL region from the WARNING region. We conclude that the path to better rare-event detection is not a different classifier but more CRITICAL train-ing data, which requires either much longer simulation runs or controlled real-world data collection.

## Per-Scenario Performance

To verify the system works across different hazard types, AUC was computed separately for each sce-nario (Table 6). The narrow spread (∆AUC < 0.002) confirms the system is robust across different traffic conditions.

Fig. 15 shows the relationship between ego vehicle speed and CRI across scenarios. Higher speeds gener-ally produce higher CRI values, with HNR scenarios showing elevated risk at lower speeds due to reduced friction. Fig. 16 shows the calibration plot, assess-ing whether CRI scores correspond to actual danger frequencies.

## Multi-Seed Robustness

To confirm results are not dependent on a single ran-dom traffic pattern, the simulation was repeated with six random seeds: the primary seed 42 (used through-out this paper) plus five additional seeds (Table 7). Seed 42 produces the most challenging traffic mix (highest proportion of dense interactions near the Atal Bridge merge zones), which accounts for its slightly lower AUC. All results reported in prior sections use seed 42 as the conservative baseline.

Table 6 Per-Scenario AUC (Physics Model)

| Scenario | AUC | n | Positives |
| --- | --- | --- | --- |
| Normal traffic | 0.9874 | 72,494 | 1,619 |
| TSV | 0.9877 | 28,246 | 653 |
| HNR | 0.9857 | 45,311 | 1,193 |

Table 7 Multi-Seed Results (N = 6). Each cell is the Tier 2 proxy AUC as a single-point estimate per seed; the seed-42 value 0.9869 is the same quantity reported as the bootstrap-median 0.989 in Table 2. The cross-seed standard deviation σ = 0.0041 quantifies stability of the simulation pipeline; it is not a substitute for the model-independent Tier 1 evaluation.

| Metric | Physics (V3.0) | XGBoost |
| --- | --- | --- |
| Seed 42 (primary) | 0.9869 | 0.7725 |
| Seeds 10–50 (mean) | 0.9971 ± 0.0007 | 0.8985 ± 0.0095 |
| All 6 seeds | 0.9954 ± 0.0041 | 0.8775 ± 0.0520 |

Fig. 15 Ego speed vs. maximum CRI, colored by scenario type (5,000 sampled rows). HNR scenarios show elevated risk at lower speeds due to reduced friction and narrow lanes.

## Ablation Study

Five configurations were tested (Table 8); F1 values are reported at the WARNING (0.60) and CRITICAL (0.80) thresholds. Including intent (γ = 0.20, A3/A4) without turn signals causes a 44% F1 drop, validat-ing γ = 0. A5 (α = 0.20, β = 0.80) is selected over A2 (α = 0.50, β = 0.50) because of better AUC rank-ordering on the Tier 2 proxy (0.9869 vs. 0.9842 on the seed-42 single-run point estimate, equivalent to the 0.989 bootstrap median reported in Table 2), despite nearly identical single-threshold F1.

Fig. 16 Calibration plot: predicted CRI vs. observed positive frac-tion. The proxy ground truth uses kinematic near-miss events (2.37% positive rate).

Table 8 Ablation Study Results

Fig. 17 Sensitivity analysis across GPS noise, packet loss, TTC threshold, alert threshold, and friction values.

coordinates into the local coordinate frame used by the physics engine. A single processing cycle with 15 simultaneous targets completes in 4.7 ms, well within the 100 ms time budget required by the SAE J2945/1 standard.

| Config | α | β | γ | Lat TTC | F1 (0.60) | F1 (0.80) | improvement with additional training data. The physics |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | 1.00 | 0.00 | 0.00 | × | 0.652 | 0.519 | model is the sole decision-maker in the current deploy- |
| A2 | 0.50 | 0.50 | 0.00 | × | 0.583 | 0.491 | ment configuration. |
| A3 | 0.35 | 0.45 | 0.20 | × | 0.328 | 0.216 | Current status: All results in this paper are from |
| A4 | 0.35 | 0.45 | 0.20 | ✓ | 0.328 | 0.216 | simulation. Hardware testing on physical vehicles is |
| A5 | 0.20 | 0.80 | 0.00 | ✓ | 0.583 | 0.494 | planned as future work. The ROS2 wrapper and ONNX |
|  |  |  |  |  |  |  | export have been validated in software-only testing. |

The XGBoost model is exported in ONNX for-mat for potential future embedded inference, pending

## Sensitivity Analysis

The system maintains strong performance under param-eter variation as shown in Fig. 17: robust with GPS noise up to 3.0 m and packet loss up to 20%.

# Real-World Deployment Pathway

Deploying this system on real vehicles requires only a standard DSRC or C-V2X On-Board Unit (OBU) that can transmit the 5-field BSM. Because the sys-tem uses only position, speed, and acceleration values, it does not need access to the vehicle’s internal com-puter network for turn signals or steering data. This makes it compatible with existing aftermarket V2V communication kits.

For embedded hardware deployment, the CRI logic has been packaged as a ROS2 node that converts GPS

# Discussion

The main finding is that the transparent physics model significantly outperforms both kinematic baselines and the exploratory XGBoost model on the same 5-field BSM input. The Tier 1 model-independent ranking

(Physics 0.971, XGBoost 0.819, TTC 0.801, Static 0.800) places the physics model clearly above every alternative with non-overlapping confidence intervals, and the paired bootstrap test rejects equality at p <

0.001. The Tier 2 proxy-based ranking is consistent

with Tier 1, ruling out the concern that the proxy is flat-

tering the model through input overlap. The severity gate — requiring at least one physical danger signal to be genuinely high before any composite score escalates

— is the architectural feature that creates this separa-tion. The ablation study (Table 8) confirms it: removing

the gate by adding the unreliable intent component (γ = 0.20) reduces F1 by 44%.

The XGBoost result, and the comparison with the

three additional learners in Section 7.6, is best read as an instructive negative result. All four data-driven clas-sifiers cluster within a narrow performance band on the easy classes and fail uniformly on the CRITICAL class. This is not a property of any one algorithm: with 41 CRITICAL training rows (SMOTE-expanded to 500), no learner has enough signal to confidently distinguish the CRITICAL region from the WARNING region. The physics model, by contrast, encodes the rare-event boundary through stopping-distance and TTC geom-etry, which is information no amount of training data could replace cheaply.

The system maintains consistent performance across the three scenario tags (∆AUC < 0.002) and across six independent random traffic seeds (σ = 0.0041, Table 7).

#### Limitations.

The evaluation is from microscopic simulation only. Real-world V2V deployment will face channel patholo-gies (multipath, ramped-up GPS error in urban canyons, BSM signing overhead) that the Gilbert–Elliott model does not capture. The validation network is a sin-gle corridor in Navi Mumbai; generalisation to other geometries is untested. The Tier 2 proxy shares inputs with the CRI score, although the Tier 1 collision-only result (Section 7.2) provides an input-independent check. The simulated fleet contains only motorised four-wheeled vehicles — pedestrians, cyclists, and two-wheelers are deliberately out of scope here because they would require V2X-Pedestrian (PSM) message handling and bicycle dynamics that the current CRI engine does not model. With turn-signal information unavailable in the 5-field BSM, the intent component is disabled in the final configuration, so a lane-change manoeuvre by the target (as opposed to the ego) is detected only through its lateral-geometric conse-quences. CRITICAL-class events remain extremely rare (54 in 146,051 rows), bounding the statistical power of any classifier evaluation at that severity level.

# Conclusion

This paper presented a V2V blind-spot detection sys-tem that operates on only five fields of the standard SAE J2735 Basic Safety Message (position, speed, acceleration, deceleration), and that converts those sig-nals into a Collision Risk Index via a severity-gated

formula. The severity gate is the principal contribution: by requiring that at least one of the deceleration-risk or TTC-risk components reach a genuinely danger-ous level before any composite score escalates, the design suppresses the false-alarm accumulation that hurts simpler weighted-sum formulations.

The physics model was evaluated over 146,051 simulated observations on a real-world road network (Atal Bridge corridor, Navi Mumbai, India). Against a model-independent collision ground truth (n = 27)

it achieved AUC = 0.971 with 95% bootstrap CI

[0.956, 0.982], statistically significantly better than the TTC kinematic baseline, the static box baseline,

and four data-driven classifiers (XGBoost, LightGBM, Random Forest, Logistic Regression). A held-out cal-ibration check confirmed that the published weights α = 0.20, β = 0.80 are not artefacts of test-set fit-ting — the train-only optimal weights generalise to

almost identical held-out test performance. SHAP anal-ysis of the exploratory XGBoost classifier confirmed that the features the data-driven model relies on (gap distance, number of targets) are the same quantities the physics CRI engine treats as primary, supporting interpretability of both routes.

#### Future work.

Four directions follow naturally from the limitations above:

- On-road validation. Deploy the same engine on an off-the-shelf DSRC or C-V2X on-board unit and val-idate against an instrumented vehicle. Real channel noise, BSM signing overhead, and GPS multipath in urban canyons cannot be evaluated in simulation.

- Vulnerable road users (VRUs). Extend the sys-tem to consume Personal Safety Messages (PSMs) for pedestrians, cyclists, and two-wheelers. This requires a modified Rdecel that does not assume vehicle-class mass and braking, plus a redesigned blind-spot zone for non-motorised actors.

- More scenarios, including static-traffic conditions. The current evaluation focuses on dynamic con-flict cases (lane-change with closing vehicles, signal violations, narrow roads). Static or quasi-static sce-narios — congestion, slow-and-stop traffic, queue tails approaching an intersection — exercise differ-ent parts of the engine (low ego speed, sustained near-zero relative velocity) and would broaden the validation envelope.

- Multi-network generalisation. Evaluate on addi-tional road geometries (motorways without merge

lanes, signalised arterials, roundabouts) and against published surrogate-safety benchmarks to demon-strate transferability beyond the single corridor used here.

A possible fifth direction, suggested by the reviewer process, is the use of a driving-simulator with human-subject participation to capture decision-making in lane-change conflicts that SUMO’s car-following mod-els approximate but do not reproduce.

The authors thank the Eclipse SUMO development team (DLR) for the open-source traffic simulation plat-form, and OpenStreetMap contributors for the Atal Bridge road network data.

Funding Declaration. The authors declare that no funding was received for this research work.

Ethics, Consent to Participate, and Consent to Publish. Not applicable.

Competing interests. The authors declare no com-peting interests.

Data Availability Statement. This study uses no proprietary, restricted, or third-party benchmark dataset. The road-network geometry of the Atal Setu corridor, Navi Mumbai, India, was obtained from OpenStreetMap (© OpenStreetMap contribu-tors, ODbL v1.0; https://www.openstreetmap.org/) on 26 September 2025 within the bounding box (lat 18.99201–19.00297° N, lon 72.84648–72.86214° E)

and converted to a SUMO microscopic-traffic net-work using netconvert v1.24.0. All vehicle trajec-tories, Basic Safety Messages (BSMs), kinematic near-miss labels, ablation tables, and trained model artifacts were generated by the Eclipse SUMO 1.19+ microscopic simulator under the deterministic random seed 42 using the open-source scripts in this repos-itory: https://github.com/omkarrr88/V2V. The com-plete synthetic dataset (Outputs/bsd_metrics.csv,

146,051 rows × 45 columns), the trained XGBoost model (Outputs/bsd_xgboost_model.json), the

alert log (Outputs/bsd_alerts.csv), and the abla-tion/sensitivity tables are committed to the same repos-itory. No human-subject data, no personal data, and no real-vehicle telemetry were used.

# References

- SAE International, “Dedicated Short Range Com-munications (DSRC) Message Set Dictionary,” SAE Standard J2735_202309, Sep. 2023.

- SAE International, “On-Board System Require-ments for V2V Safety Communications,” SAE Standard J2945/1_201603, Mar. 2016.

- IEEE, “IEEE Standard for Information Technol-ogy — Wireless Access in Vehicular Environ-ments,” IEEE Std 802.11p-2010, Jul. 2010.

- 3GPP, “Study on LTE-based V2X Services,” 3GPP TR 36.885 V14.0.0, Jun. 2016.

- Euro NCAP, “Lane Change Assist System (LCAS) Test Protocol,” Version 1.0, 2019.

- AASHTO, A Policy on Geometric Design of Highways and Streets, 7th ed. Washington, DC, 2018.

- E. N. Gilbert, “Capacity of a burst-noise channel,” Bell Syst. Tech. J., vol. 39, no. 5, pp. 1253–1265, Sep. 1960.

- T. Chen and C. Guestrin, “XGBoost: A scalable tree boosting system,” in Proc. ACM SIGKDD, San Francisco, CA, Aug. 2016, pp. 785–794.

- P. A. Lopez et al., “Microscopic traffic simulation using SUMO,” in Proc. IEEE ITSC, Maui, HI, Nov. 2018, pp. 2575–2582.

- World Health Organization, “Global Status Report on Road Safety 2023,” WHO, Geneva, Dec. 2023.

- MoRTH, “Road Accidents in India 2022,” Gov-ernment of India, New Delhi, Oct. 2023.

- A. Mousa et al., “Vehicle-to-vehicle communica-tion for blind spot detection,” IEEE Trans. Intell. Transp. Syst., vol. 22, no. 5, pp. 3120–3131, May

2021.

- A. Festag, “Cooperative intelligent transport sys-tems standards in Europe,” IEEE Commun. Mag., vol. 52, no. 12, pp. 166–172, Dec. 2014.

- S. Biswas, R. Tatchikou, and F. Dion, “Vehicle-to-vehicle wireless communication protocols for enhancing highway traffic safety,” IEEE Commun. Mag., vol. 44, no. 1, pp. 74–82, Jan. 2006.

- N. Chawla et al., “SMOTE: Synthetic minority over-sampling technique,” J. Artif. Intell. Res.,

vol. 16, pp. 321–357, 2002.

- B. Kim et al., “An IMM/EKF approach for enhanced multitarget state estimation for inte-grated threat assessment,” IEEE Trans. Veh. Technol., vol. 64, no. 3, pp. 876–889, Mar. 2015.

- Z. A. Rawashdeh and S. M. I. Mahmud, “A novel V2V-based blind spot detection system,” in Proc. IEEE VTC-Fall, Calgary, AB, Sep. 2008, pp. 1–5.

- S. M. Lundberg and S.-I. Lee, “A unified approach to interpreting model predictions,” in Proc. NeurIPS, Long Beach, CA, Dec. 2017,

pp. 4765–4774.

- R. Xu, H. Xiang, X. Xia, X. Han, J. Li, and

J. Ma, “OPV2V: An open benchmark dataset and fusion pipeline for perception with vehicle-to-vehicle communication,” in Proc. IEEE ICRA, Philadelphia, PA, May 2022, pp. 2583–2589.

- R. Xu, H. Xiang, Z. Tu, X. Xia, M.-H. Yang, and

J. Ma, “V2X-ViT: Vehicle-to-everything coopera-tive perception with vision transformer,” in Proc. ECCV, Tel Aviv, Israel, Oct. 2022, pp. 107–124.

- R. Xu et al., “V2V4Real: A real-world large-scale dataset for vehicle-to-vehicle cooperative percep-tion,” in Proc. IEEE CVPR, Vancouver, BC, Jun. 2023, pp. 13712–13722.

- Y. Li, D. Ma, Z. An, Z. Wang, Y. Zhong, S. Chen, and C. Feng, “V2X-Sim: Multi-agent collab-orative perception dataset and benchmark for autonomous driving,” IEEE Robot. Autom. Lett., vol. 7, no. 4, pp. 10914–10921, Oct. 2023.

- A. Caillot, S. Ouerghi, P. Vasseur, R. Boutteau, and Y. Dupuis, “Survey on cooperative percep-tion in an automotive context,” IEEE Trans. Intell. Transp. Syst., vol. 23, no. 9, pp. 14204–14223,

Sep. 2022.

- S. Liu, C. Gao, Y. Chen, X. Peng, X. Kong,

K. Wang, R. Xu, W. Jiang, H. Xiang, J. Ma, and J. Ma, “Towards vehicle-to-everything autonomous driving: A survey on collaborative perception,” IEEE Trans. Intell. Transp. Syst., vol. 25, no. 5, pp. 3614–3633, May 2024.

- K. Garlichs, M. Wegner, and L. C. Wolf, “Design-ing C-V2X for connected and automated driving,” IEEE Veh. Technol. Mag., vol. 18, no. 3, pp. 46–55, Sep. 2023.